Decide on an IoT device

By Haydn Roberts

Going into this project, 3 main IoT devices were pitched as potential options for this project, Nordic semiconductors, Arduino and Raspberry Pi, this will be research into the options to evaluate what would be ideal for the group to work with.
specifically the Nordic nRF9160, Arduino MKR NB1500 and the Pi w Pico / Pi zero 2 w seem to be the most ideal offerings from each company.

Languages:
All three devices can be programmed in C/C++, which aligns with the team’s experience. The Raspberry Pi also supports Python and Node.js, offering faster prototyping options for the app-connected system. The Nordic is the most complex to develop however comes with an online course that teaches how to use it alongside its development kit.

Power consumption:
This is a more complex topic and lead me down many ‘rabbit holes’ that gave me more questions than answers. It did lead me to realise that power consumption is a more important matter than the efficiency of the device as they will need to keep charge for multiple hours/days. This research shows that the Pi zero w 2 is around 5 times faster at complex calculations however it takes up more power which would not be ideal for this project: https://andrejacobs.org/b24/electronics/raspberry-pi-zero-2-w-temperature-and-power-consumption
The Pi zero 2 w draws around 200-500 mAh at idle while the Pi Pico W draws around 20-40 mAh, making it the better option of the two

The Nordic nRF9160 has the lowest power consumption during cellular and GNSS operation due to its optimized LTE-M/NB-IoT modem and deep-sleep states, making it the most power-efficient for continuous location tracking, however the Pi Pico W is also viable for this as using the formula displayed bellow, it could last around 75-150 hours per AA battery as a AA battery has around 3000mAh and the Pico idles at around 20-40mAh: https://www.reddit.com/r/raspberry_pi/comments/100aq5g/how_long_can_you_power_pico_w_using_batteries/ 
The Arduino MKR NB1500 has an integrated u-blox modem but suffers from high transmit currents (60–200 mA) during communication, limiting battery life.

Size:
All the devices are very small and would fit in a pocket without any issues. The Pico W is the smallest device though at 21 × 51 mm

Cost:
The Nordic nRF9160 is £25 to £30 for the SiP and can cost up to £100 for the full development kit that would be needed for this project, this is not a cost-effective approach. The SiP could be placed in custom PCB however this makes the Nordic even more complex to develop with.
The Raspberry Pi Pico is £4 to $10 making it very cost effective and a valid option for this project. The Pi zero W 2 costs around £15 to £20.
The Arduino MKR NB1500 Is around £70 with cheaper alternatives like the MKR Zero costing £28

Conclusion:
The Arduino is not suitable for this project due to its poor power consumption and lack of a budget purchase option.
The Nordic nRF9160 is a Great product, however is not a suitable price for this project,  and does not offer enough of an improvement from the Raspberry Pi to be chosen.
Overall, the Raspberry Pi Pico W has the best programming language options, great battery life, a small size and a very low price, making it the ideal IoT device for this project.

The main downside of the Pico W is that it will require an external GPS module to be attached to it, this should be researched in the data transmission and GPS section of the research and planning.